Task 4

Cornell university

1. ODLC Pipeline

General Pipeline:

The drone takes photos from above using a Sony R10C camera mounted on a 2-axis gimbal to keep the camera pointing straight down at the ground.

The photos are sent down to a ground server, where an automated system (AVS) runs Mask R-CNN to spot possible targets in the image.

Once a target is found, it uses U-Net to isolate the target shape, and custom code extracts its color, shape, and letters/numbers.

A human operator on the ground checks the detections on a dashboard, fixes any mistakes or duplicate tags, and sends the final target coordinates to the autopilot for the drop mission.

Localization:

How it works: They take the plane's GPS position, altitude, and tilt angles (roll/pitch/yaw) from the flight controller at the exact moment the photo was taken, and project where the pixels inside the image land on the ground.

The clever hardware trick: Usually, camera shutter delay messes up the coordinates. To fix this, they built a custom circuit board that senses the tiny electrical current spike when the camera shutter physically clicks. This logs the exact millisecond the picture was taken and matches it to the plane’s sensor data.

Accuracy: Because of the gimbal stabilization and exact time-matching, their average position error was only about 7 feet.

2. Communication Systems

The Communication Setup:

They split the connection into two separate channels so the heavy image files don't lag the flight controls:

900 MHz link: Used for MAVLink flight data, drone telemetry, and basic commands.

5.8 GHz Wi-Fi link: Used solely to stream full-size photos from the plane to the ground team.

Wi-Fi Details:

They put a Ubiquiti Rocket AC radio on the plane and pointed an antenna-tracker with a Ubiquiti NanoStation at it from the ground.

This gave them a solid 100 Mbps connection with a range of up to 1 km and almost zero dropped packets.

Onboard Computer & Connection:

They used a Raspberry Pi 4B inside the plane.

The Pi connects directly to the Rocket AC radio, bridging it over Wi-Fi to the ground router. This puts the onboard Pi, the pilot's laptop, and the vision server all on the same local network.